Text for tenders

Pressure steamer VitalityEVO - Pro

| Pos.: | Description | Quantity | Unit price | Total |
| --- | --- | --- | --- | --- |

|  | Execution: | Planned: | Offered: |
| --- | --- | --- | --- |
|  | Device type: | Table unit |  |
|  | Capacity: | 3 x GN 1/1- 65 mm |  |
|  |  | 2 x GN 1/1 - 100mm 1 x GN 1/1 - 200mm |  |
|  | Insertion distance: | 67 mm |  |
|  | Dimensions (L x D x H): | 624 x 843,5 x 866 mm |  |
|  | Weight: | Net ca. 95 kg |  |
|  | Connected load Electrical: | 16,0 kW |  |
|  | Voltage: | 400 V 3N 50 Hz |  |
|  | Fuse protection: | 3 x 25 A |  |
|  | Water inlet: | 1 x G ¾ |  |
|  | Water drain: | 20 mm |  |
|  | Cable length: | 1.5 m |  |
|  | Test and safety symbol: | CE |  |
|  | Equipment protection class: | IP X5 |  |
|  | Design: | DIN 18866 |  |

Specifications

|  | Heat exchanger | No |  Yes |  |
| --- | --- | --- | --- | --- |
|  | Double-walled safety pressure door | No |  Yes |  |
|  | Integrated water softener | No |  Yes |  |
|  | Connection to power optimization inclusive | No |  Yes |  |
|  | Touch screen | No |  Yes |  |
|  | Cooking programs | No |  Yes |  |
|  | Core probe sensor | No |  Yes |  |
|  |  | No |  Yes |  |
|  |  | No |  Yes |  |

|  |  | Total: |  |
| --- | --- | --- | --- |
|  | Manufacturer: |  |  |
|  | Model: |  |  |

| Pos.: | Description | Quantity | Unit price | Total |
| --- | --- | --- | --- | --- |

Construction

- Compact device, robust design, frame completely made of stainless steel.

- Floor level side panel with rubber seal for hygienic installation according to EN1672.

- Integrated water softener with electronic hardness adjustment and forced regeneration.

- Heat exchanger to reduce energy and water consumption.

- Cooking chamber, steam generator and heat exchanger completely insulated.

- Double-walled door with magnetic safety lock.

- Removable rack in heavy-duty design.

- Installation on base frames or work surfaces

- High-performance steam generator enables short heat-up times, fast steam readiness, unrivalled simple dry steam injection, continuous live steam

- Exhaust steam extinguisher integrated in the unit

Data exchange

- Internal HACCP recording with export via USB

- Serial USB interface for data transfer.

- Connection to power optimization system included

Panel

- High-resolution touch screen and state-of-the-art electronic control.

- Time input from 10 seconds to 24 hours.

- Automatic start time preselection with 24h pre-programming.

- 300 program memory places à 20 phases with approx. 80 pre-programmed cooking programs.

- Easy programming or addition via numeric keypad.

- Simple search of stored cooking programs.

- Favourites with up to 20 of the most important cooking programs.

- Category Selection of cooking programs

- The core temperature sensor in the 20-99°C range supports food safety in production.

- Special programmes DeltaT, soft cooking and long-term cooking

- Control panel on top, touch panel with large, easy-to-read digital displays

- Infinitely variable, temperature input accurate to the degree from 50 - 119°C with automatic pressure adjustment

- Cooking ranges:

  - Pressure-free / low temperature 50 - 95 °C (also for long-term cooking up to 24h)

  - Pressure cooking / saturated steam 96 - 119 °C (up to 1 bar gauge pressure)

- AQA - automatic quantity adjustment to the quantity of food to be cooked

- Electronically adjustable height adjustment (above sea level)

- Integrated fault diagnosis system with technician pop-up function

Cleaning & Energy saving

- Simple hygiene due to smooth surfaces and round cooking chamber.

- Retractable spray with automatic return to simplify cooking and cleaning processes.

- Energy saving mode, display dark after 15 min without input

- Software-supported rinsing function for simplified manual cleaning

| Pos.: | Description | Quantity | Unit price | Total |
| --- | --- | --- | --- | --- |

Execution

- Door seal removable and simply plugged in.

- Condensate collecting integrated

- Thermostatically controlled vapour extinguishing.

- Quick door lock for one-hand operation.

- Includes connection to energy optimisation system.

- Installation kit included: Certified inlet and outlet hose for water installation and vent funnel.

Optional equipment and system accessories

  - Special voltages

  - Ethernet interface

  - Booster pump

  - Base frames

Technical changes reserved!

Manufacturer: Salvis AG, Nordstrasse 15, 4665 Oftringen, Switzerland