Product description

Condensation hood for Salvis CucinaEVO 623T

Specification:

| Type: | Condensation hood |  |
| --- | --- | --- |
| Dimensions (W x D x H): | 550 x 648 x 150 |  |
| Connected load: | 0,06 kW |  |
| Voltage: | 230 V |  |
| Fan speeds | 4 |  |
| Extraction capacity | 600 m3/h |  |
| Noise level | 48,5 dBA (2) – 58,0 dBA (4) |  |
| Protection: | Water jet protection IPx4 |  |
| Max. energy consumption | 0,06 kW/h |  |

Specific characteristics:

- Compact mounted unit for Salvis CucinaEVO 623T.

- Combination of hood and steam condenser.

- There is a minimal overlap from the outer contours to the Salvis CucinaEVO.

- Control and power supply via Salvis CucinaEVO.

- Fan level 1 switches on automatically when Salvis CucinaEVO starts

- Four-level blowing power is controlled automatically, dependent on the operating level of the Salvis CucinaEVO

- During cooking, steam and vapours condense and are removed by 4 heat exchangers

- Condensate drains directly through the drainage system of the Combi-Steamer

- Fan run-on controlled automatically by a timed relay

- Condensation effectiveness of hood system exceeds 80 %.

Execution / Specification:

- Housing and plate capacitors made of chrome-nickel steel AISI 304

- Surface smoothed

- Condensation hood with removable, dishwasher-safe grease filters

Subject to technical changes!

Manufacturer: Salvis AG, Nordstrasse 15, 4665 Oftringen, Switzerland