Text for tenders

Combi-Steamer Salvis CucinaEVO 1011QT-Pro

| Item: | Description | Quantity | ME | Unit price | Total price |
| --- | --- | --- | --- | --- | --- |

|  | Design: | Planned: | Quoted: |
| --- | --- | --- | --- |
|  | Type: | Tabletop unit |  |
|  | Capacity: | 10+1 x GN1/1 |  |
|  |  | 20+2 x GN1/1 for banquet and low-temperature cooking |  |
|  | Spacing: | 67 mm |  |
|  | Dimensions (W x D x H) | 980 x 795 x 1120 mm |  |
|  | Weight: | Approx. 160 kg net |  |
|  | Connected load: | 16.4 kW |  |
|  | Voltage: | 400 V 3N 50/60 Hz |  |
|  | Fuse(s): | 3 x 25 A |  |
|  | Water connection: | 2 x G ¾ A |  |
|  | Water drainage: | 50 mm |  |
|  | Direct connection – intrinsically safe to DVWG: | Yes |  |
|  | Approvals: | SVGW, CE |  |
|  | Protection rating: | Water jet protection IPX5 |  |
|  | Type of equipment: | DIN 18866 |  |

Specification

|  | Triple door glazing | No |  Yes |  |
| --- | --- | --- | --- | --- |
|  | Integrated heat recovery | No |  Yes |  |
|  | Cleaning system with 4 steps | No |  Yes |  |
|  | Cleaning system can be selected  Powder  Liquid | No |  Yes |  |
|  | Sideways insertion GN 1/1 | No |  Yes |  |
|  | Lengthways insertion GN 1/1, GN 2/1, BN | No |  Yes |  |
|  | 1 removable rack for all sizes | No |  Yes |  |
|  | Cooking chamber size - standard GN 2/1 | No |  Yes |  |
|  | Touchscreen | No |  Yes |  |
|  | Energy-certified according to ENAK | No |  Yes |  |
|  | Integrated cookbook | No |  Yes |  |
|  | Door opening | Right |  Left |  |

|  |  | Subtotal: |  |
| --- | --- | --- | --- |
|  | Brand: |  |  |
|  | Model: |  |  |

| Item: | Description | Quantity | ME | Unit price | Total price |
| --- | --- | --- | --- | --- | --- |

Key features

- High-resolution TFT colour touchscreen integrated flush into the hygienic glass panel

- Simple, easy-to-understand user guidance - operation and display in one element.

- Temperature, humidity, time or core temperature changeable by scrolling or using the numeric keypad.

- Temperature input accurate to the degree from 30 °C to 250 °C, humidity adjustable from 0 to 100 %.

- Active climate - dry and humid heat can be freely combined with active humidification and dehumidification for an optimum cooking cabinet climate.

- Inherent humidity control in 5 levels in the hot air climate

- Special programmes NT cooking, soft cooking, cook and hold - for long-term cooking from 30 °C to 120 °C.

- ∆-T cooking - core temperature-dependent controlled cooking processes and insertion times for layer monitoring.

- Combi cooking as an extended programme function for cooking cabinet capacity utilisation.

- Baking mode with entry of the amount of water in ml and stand resting times of the fan wheel, as well as post-baking function up to 5 min

- With the CombiStep function, up to 20 different cooking steps can be combined as required and clearly displayed in the cooking step overview.

- Extensive memory function for entering your own individual cooking programmes according to programme lists, categories, info texts and image assignment

- Eco Mode for energy-optimised cooking processes such as slow cooking and braising

- 5 fan speeds plus cycle function for perfect, even cooking and baking results.

- Fan wheel with auto reverse mode for high efficiency and timed for gentle cooking.

- Temp-Activ - automatic preheating and holding at the pre-set temperature.

- Cool down function, motorised when the door is open, for efficient climate control.

- Start time preselection and E/2 function (energy-saving function).

- Automatic quantity adjustment - AQA.

- Cooking cabinet safety steam extraction SDA.

- Manual humidification - for the individual addition of moisture, e.g. when baking.

- Time setting 1 min. to 24 hours, continuous operation and start time preselection up to 24 hours in real time.

- Start from any phase, for multi-phase cooking processes, more convenience in use

- Core temperature control with multi-point probe - for precise measurement of the core temperature from 0° to 99 °C,

- Data exchange via USB.

- HACCP Data Recording - automatic recording of HACCP/food hygiene regulation (LMHV) data from the last 300 processes

- Auto-diagnosis system with operating, error and warning indicators.

- Service pop-up window to check current technical parameters live.

- Default cooking parameters can be customised for the type of operation and use

- Time setting from 1 minutes to 24 hours, continuous operation and start time pre-setting up to 24 hours in real-time.

- Extensive memory function to enter your own custom cooking programs with program lists, categories, info texts and linked images.

- Search function to find saved cooking programs more easily.

- Energy-saving mode - light switches off automatically after 15 minutes of inactivity

- Water filter counter - indicates when the filter should be changed for long-term maintenance of value

- System-supported manual cleaning process - to meet hygiene requirements

- Vitalisation with pre-set parameters for single portions, GN trays and banquet system

- Start/stop button with function indicator for running (red), idle (green) and stopped (blue)

- Memory - repeat last entered program without re-entering it.

| Item: | Description | Quantity | ME | Unit price | Total price |
| --- | --- | --- | --- | --- | --- |

Version

- Hygienic cooking chamber, GN 2/1 size as standard, with rounded corners at top and bottom - seamlessly welded on all sides with central drain and drain strainer

- 4-in-1 flexible slide-in loading system for GN2/1 lengthwise and GN1/1 crosswise,

GN1/1 lengthwise and baking standard

- Steam-Eco system for direct, on-demand steam generation in the cooking chamber

- Inner and outer housing made of chrome-nickel steel 18/10 (material No. 1.4301)

- Panorama viewing door with lock-in position

- Triple door glazing for minimum heat loss

- Removable, easy to insert door seal

- Condensate collection channel integrated onto housing with drain plus separate door condensate collection channel.

- Thermostatically controlled vapour elimination

- Quick door close for one-hand operation

- Ventilation shutdown when the door is open using non-contact magnetic switch for energy optimisation and reliability

- Integrated, pull-out hose with spray with automatic retraction and water shutoff

- Halogen interior lighting - clear for visual inspection

- Two separate connections for hard and soft water - intrinsically safe according to SVGW

- Eco-Autoclean - automatic cleaning system with contactless powder cleaner addition via bulk dosing bottle

- Heat recovery in the counter-flow heat exchanger utilises the energy from exhaust steam and condensate for energy savings of up to 30%.

- Ethernet interface

Optional equipment and system accessories

- Eco-Autoclean liquid cleaner

- Sous-vide programs including sensor

- Fat filter

- Special voltages

- Banquet systems

- Ship Model

- Condensation hood with removable, dishwasher-safe grease filters

Subject to technical changes!

Manufacturer: Salvis AG, Nordstrasse 15, 4665 Oftringen, Switzerland