Text for tenders

Combi-Steamer Salvis CucinaEVO 2021QT-Pro

| Item: | Description | Quantity | ME | Unit price | Total price |
| --- | --- | --- | --- | --- | --- |

|  | Design: | Planned: | Quoted: |
| --- | --- | --- | --- |
|  | Type: | Standalone |  |
|  | Capacity: | 20 x GN 2/1 |  |
|  | Spacing: | 67 mm |  |
|  | Dimensions (W x D x H) | 1115 x 895 x 1925 mm |  |
|  | Weight: | Approx. 360 kg net |  |
|  | Connected load: | 51.5 kW / 67.1 kW |  |
|  | Voltage: | 400 V 3N 50/60 Hz |  |
|  | Fuse(s): | 3 x 80/100 A |  |
|  | Water connection: | 2 x G ¾ A |  |
|  | Water drainage: | 50 mm |  |
|  | Direct connection – intrinsically safe to DVWG: | Yes |  |
|  | Approvals: | SVGW, CE |  |
|  | Protection rating: | Water jet protection IPx4 |  |
|  | Type of equipment: | DIN 18866 |  |

Specification

|  | Integrated heat recovery | No |  Yes |  |
| --- | --- | --- | --- | --- |
|  | Cleaning system with 4 steps | No |  Yes |  |
|  | Cleaning system can be selected  Powder  Liquid | No |  Yes |  |
|  | Sideways insertion GN 1/1 | No |  Yes |  |
|  | Lengthways insertion GN 2/1, BN | No |  Yes |  |
|  | Folding cooking chamber sealing panel | No |  Yes |  |
|  | Cooking chamber size - standard GN 2/1 | No |  Yes |  |
|  | Touchscreen | No |  Yes |  |
|  | Energy-certified according to ENAK | No |  Yes |  |
|  | Integrated cookbook | No |  Yes |  |
|  | Door opening | Right |  Left |  |

|  |  | Subtotal: |  |
| --- | --- | --- | --- |
|  | Brand: |  |  |
|  | Model: |  |  |

| Item: | Description | Quantity | ME | Unit price | Total price |
| --- | --- | --- | --- | --- | --- |

Key features

- High-resolution TFT colour touch screen, fully integrated in the hygienic glass panel

- Easy, understandable user interface - controls and display in one place.

- Temperature, humidity, time and core temperature can be changed with the scrollbar or the ten-key keypad

- Temperature input accurate to the degree from 30 °C to 250 °C, humidity adjustable from 0 to 100 %.

- Active climate - dry and moist heat with active moistening and dehumidification in any combination

- Inherent moisture control at 5 levels in hot air environment

- Special programs: low-temperature cooking, Softcooking, Cook and Hold - for slow-cooking from 30 °C to 120 °C.

∆-T cooking - cooking processes and feed rates controlled depending on core temperature to monitor supports

- Combi cooking as advanced program function for exploiting cooking chamber capacity.

- Baking mode with setting for water quality in ml and idle times for ventilator, as well as rebaking function up to 5 minutes.

- The CombiStep function allows you to combine up to 20 different cooking steps, which are clearly displayed in the cooking steps overview.

- Eco Mode to optimise power consumption of cooking processes, such as slow-cooking and stewing

- 5 ventilator speeds plus interval mode for perfect, uniform cooking and baking results.

- Highly effective ventilator with auto-reverse and interval mode for gentle cooking.

- Temp active - automatic preheating or cooling down to pre-set temperature.

- Cool down function with motor control when door is open for efficient climate control.

- Start time pre-set and E/2 function (energy saving function).

- Automatic quantity adjustment (AQA).

- Cooking chamber safety steam extraction (SDA)

- Manual steam injection - for custom addition of moisture e.g. for baking.

- Time setting from 1 minutes to 24 hours, continuous operation and start time pre-setting up to 24 hours in real-time.

- Start from any phase for multi-phase cooking processes for greater convenience

- Core temperature regulation with multi-point sensor - to accurately measure the core temperature from 0 °C to 99° C; control cooking processes with warning if failed to insert probe.

- Data exchange via USB.

- HACCP data recording - automatic recording of HACCP  data from the last 300 processes

- Default cooking parameters can be customised for the type of operation and use

- Energy-saving mode - lights switches off automatically after 15 minutes of inactivity

- Water filter counter - indicates when the filter should be changed for softening systems for long-term maintenance of value

- System-supported manual cleaning process - to meet hygiene requirements

- Regeneration with pre-set parameters for single portions, GN trays and banquet system

- Start/stop button with function indicator for running (red), idle (green) and stopped (blue)Water filter counter - indicates when the filter should be changed for long-term maintenance of value

- System-supported manual cleaning process - to meet hygiene requirements

| Item: | Description | Quantity | ME | Unit price | Total price |
| --- | --- | --- | --- | --- | --- |

Version

- Hygienic cooking chamber, GN 2/1 size as standard, with rounded corners at top and bottom - seamlessly welded on all sides with central drain and drain strainer

- Steam-Eco system for direct, on-demand steam generation in the cooking chamber

- Inner and outer housing made of chrome-nickel steel 18/10 (material No. 1.4301)

- Panorama viewing door with lock-in position

- Triple door glazing for minimum heat loss

- Removable, easy to insert door seal

- Condensate collection channel integrated onto housing with drain plus separate door condensate collection channel.

- Thermostatically controlled vapour elimination

- Quick door close for one-hand operation

- Ventilation shutdown when the door is open using non-contact magnetic switch for energy optimisation and reliability

- Integrated, pull-out hose with spray with automatic retraction and water shutoff

- Halogen interior lighting - clear for visual inspection

- Two separate connections for hard and soft water - intrinsically safe according to SVGW

- Cooking chamber sealing panel for sealing during pre-heating and cleaning without slide-in trolley

- Eco-Autoclean - automatic cleaning system with contactless powder cleaner addition via bulk dosing bottle

- Heat recovery in the counter-flow heat exchanger utilises the energy from exhaust steam and condensate for energy savings of up to 30%.

- Ethernet interface

- Connection to energy optimisation system

- 20 x GN 2/1 roll-in trolley included

Optional equipment and system accessories

- Eco-Autoclean  liquid cleaner

- Sous-vide programs including sensor

- Program memory functions

- Fat filter

- Special voltages

- Banquet systems

- Condensation hood with removable, dishwasher-safe grease filters

Subject to technical changes!

Manufacturer: Salvis AG, Nordstrasse 15, 4665 Oftringen, Switzerland