Text for tenders

Combi-Steamer Salvis CucinaEVO 623T-Easy

| Item: | Description | Quantity | ME | Unit price | Total price |
| --- | --- | --- | --- | --- | --- |

|  | Design: | Planned: | Quoted: |
| --- | --- | --- | --- |
|  | Type: | Tabletop unit |  |
|  | Capacity: | 6 x GN 2/3 |  |
|  | Spacing: | 60 mm |  |
|  | Dimensions (W x D x H) | 550 x 629 x 764 mm |  |
|  | Weight: | Approx. 70 kg net |  |
|  | Connected load: | 3.6 kW / 5,8 kW |  |
|  | Voltage: | 230 V N 50/60 Hz / 400 V 3N 50/60 Hz |  |
|  | Fuse(s): | 1 x 16 A / 3 x 16 A |  |
|  | Water connection: | 2 x G ¾ A |  |
|  | Water drainage: | 40 mm |  |
|  | Direct connection – intrinsically safe to DVWG: | Yes |  |
|  | Approvals: | SVGW, CE |  |
|  | Protection rating: | Water jet protection IPX4 |  |
|  | Type of equipment: | DIN 18866 |  |

Specification

|  | Triple door glazing | No |  Yes |  |
| --- | --- | --- | --- | --- |
|  | Cleaning system with 4 steps | No |  Yes |  |
|  | Cleaning system can be selected  Powder  Liquid | No |  Yes |  |
|  | Lengthways insertion GN 1/1, GN 1/2 | No |  Yes |  |
|  | Touchscreen | No |  Yes |  |
|  | Energy-certified according to ENAK | No |  Yes |  |
|  | Door opening | Right |  Left |  |

|  |  | Subtotal: |  |
| --- | --- | --- | --- |
|  | Brand: |  |  |
|  | Model: |  |  |

| Item: | Description | Quantity | ME | Unit price | Total price |
| --- | --- | --- | --- | --- | --- |

Key features

- High-resolution TFT colour touch screen, fully integrated in the hygienic glass panel

- Easy, understandable user interface - controls and display in one place.

- Temperature, humidity, time and core temperature can be changed with the scrollbar or the ten-key keypad

- Temperature adjustment from 30 °C up to 250° C, humidity from 0 to 100%.

- Active climate - dry and moist heat with active moistening and dehumidification in any combination

- Inherent moisture control at 5 levels in hot air environment

- The CombiStep function allows you to combine up to 20 different cooking steps, which are clearly displayed in the cooking steps overview.

- Eco Mode to optimise power consumption of cooking processes, such as slow-cooking and stewing

- 5 ventilator speeds plus interval mode for perfect, uniform cooking and baking results.

- Highly effective ventilator with auto-reverse and interval mode for gentle cooking.

- Temp active - automatic preheating or cooling down to pre-set temperature.

- Cool down function with motor control when door is open for efficient climate control.

- Start time pre-set and E/2 function (energy saving function).

- Automatic quantity adjustment (AQA).

- Cooking chamber safety steam extraction (SDA)

- Manual steam injection - for custom addition of moisture e.g. for baking.

- Time setting from 1 minutes to 24 hours, continuous operation and start time pre-setting up to 24 hours in real-time.

- Start from any phase for multi-phase cooking processes for greater convenience

- Core temperature regulation with multi-point sensor - to accurately measure the core temperature from 0 °C to 99° C; control cooking processes with warning if failed to insert probe.

- Technical data exchange via USB.

- HACCP Data Recording - automatic recording of HACCP/food hygiene regulation (LMHV) data from the last 300 processes

- System-supported manual cleaning process - to meet hygiene requirements

- Auto-diagnosis system with operating, error and warning indicators.

- Service pop-up window to check current technical parameters live.

- Default cooking parameters can be customised for the type of operation and use

- Energy-saving mode - lights switches off automatically after 15 minutes of inactivity

- Water filter counter - indicates when the filter should be changed for softening systems for long-term maintenance of value

- Regeneration with pre-set parameters for single portions, GN trays and banquet system

- Start/stop button with function indicator for running (red), idle (green) and stopped (blue)

| Item: | Description | Quantity | ME | Unit price | Total price |
| --- | --- | --- | --- | --- | --- |

Version

- Hygienic cooking chamber with rounded corners at the top and bottom - seamlessly welded on all sides and central drain and drain sieve

- GN 2/3  lengthwise insert

- Steam-Eco system for direct, on-demand steam generation in the cooking chamber

- Inner and outer housing made of 18/10 stainless steel - material no. 1.4301

- Panorama viewing door with latching position

- Triple door glazing with infrared mirroring for minimised heat radiation

- Door seal removable and simply plugged in

- Condensate collection channel integrated into the housing with drain plus separate door condensate collection channel.

- Thermostatically controlled vapour extraction

- Quick-release door lock for one-handed operation

- Fan switch-off when door is opened via contactless magnetic switch for energy optimisation and operational safety

- Integrated pull-out hose spray with automatic retraction and water stop

- Stainless steel rack for flexible use of GN containers/grids

- Halogen interior lighting - precise visual control

- Two separate connections for cold and soft water - intrinsically safe according to SVGW

- Ethernet interface

Optional equipment and system accessories

- Eco-Autoclean – EAC in two version, for powder or liquid cleaner

- Program operation/cookbook including favourites and categories

- Special programs

- Sous-vide programs including sensor

- Connection to energy optimisation system (EOS)

- Fat filter

- Baking shelf rack, rack spacing: 75 mm

- Special voltages

- Ship model

- Heat exchanger

- Condensation hood with removable, dishwasher-safe grease filters

Subject to technical changes!

Manufacturer: Salvis AG, Nordstrasse 15, 4665 Oftringen, Switzerland