| # QUOTATION TEXT |  |
| --- | --- |
|  | Range Master Compact Induction 4 zone full surface |
|  | Free standing unit |
|  | - Compact external dimensions - Freestanding or wall-mounted installation - Storage compartment accessible from the front - operable from the front - Even heat distribution - Short heat-up time - With energy-saving pan detection |

Special features:

- Output 28 kW (2 x 14 kW)

- Low radiant heat

- 4 full-surface induction coils 260 x 135mm- Ceramic plate 720 x 650mm

- 4 height-adjustable stainlees steel feet from 894 - 930mm

- Incl. connection cable 1.5m

Ausführung:

- Induction hob with 6 mm thick ceramic plate and innovative large rectangular coils,

ideal for à la carte cooking.

- Even the smallest pans can be used on the round coils.

- Flush-fitting, seamless glass ceramic hob with electronic pan detection to save energy.

- Simple and ergonomic knob operation.

- Electronic temperature control and pan detection.

- When the cooking equipment is placed on or removed, the heating power of the appliance switches on and off automatically.

- The infinitely variable setting guarantees sensitive control of the four separately adjustable cooking zones.

- Large knobs, easy-to-understand symbols, scales and control elements on the ergonomically

control panel make work easier.

- The ceramic hob is glued flush into the cover, making it easy to clean and offering a high degree of hygiene.

- Open substructure, accessible from the front, can be operated in Hygiene Standard with one

- adjustable shelf

- Cover in 2 + 2 mm, A-40 on all sides

Technische Daten

| Aussen B x T x H | 800 x 800 x 750/900mm |
| --- | --- |
| Glasgrösse: Spulengrösse: Anzahl Heizzonen Abstellraum offen: | 720 x 650mm 160 x 135mm 4 Zonen à 2 Spulen 320/378 x 759 x 468mm |
| Anschlusswert | 400 V / 3~N / 28 kW (2 x 14 kW) / 2 x (3 x 20A) |
| Prüf- und Sicherheitszeichen | CE |
| Geräteschutzart | IPX4 |

Technische Änderungen vorbehalten!