Tender text

Cooker Master Compact – Ceramic hob with pan detection

| Pos.: | Bezeichnung | Menge | ME | Einzelpreis | Gesamtpreis |
| --- | --- | --- | --- | --- | --- |

|  | Ausführung: | Geplant: | Angeboten: |
| --- | --- | --- | --- |
|  | Type: | Free-standing unit |  |
|  | Dimensions (L x W x H):: | 800 x 800 x 750/900mm |  |
|  | Dimensions Glass (L x W): | 720 x 650mm |  |
|  | Glass thickness: | 6 mm |  |
|  | Substructure: | Storage room accessible from the front |  |
|  | Substructure Dimensions: | 540/598 x 759 x 468mm |  |
|  | Hygiene: | Standard |  |
|  | Substructure equipment: | Shelf, 3-way adjustable |  |
|  | Heating zone size: | 300 x 300mm |  |
|  | Number of heating zones: | 4 |  |
|  | Electrical connected load: | 16,0 kW |  |
|  | Voltage: | 400 V 3N 50/60 Hz |  |
|  | Fuse protection: | 2x (3 x 20 A) |  |
|  | Test and safety mark: | CE |  |
|  | Device protection type: | IPX4 |  |
|  | Sensitive heat emission: | 3,2 kW |  |
|  | Heat output latent: | 1,28 kW |  |
|  | Vapour emission: | 1,89 kg/h |  |

Spezifications

|  | 6mm glass stop720 x 650mm | No |  Yes |  |
| --- | --- | --- | --- | --- |
|  | Flush glass installation | No |  Yes |  |
|  | Square coils 300 x 300mm | No |  Yes |  |
|  | Electronic pan detection | No |  Yes |  |
|  | Thermostatic control | No |  Yes |  |
|  | Worktop 2mm | No |  Yes |  |
|  | 2mm CNS reinforcement profiles | No |  Yes |  |
|  | 4 height-adjustable CNS feet from 894 - 930mm | No |  Yes |  |
|  | Connection cable 1.5m | No |  Yes |  |
|  | Visible sides in microlon finish | No |  Yes |  |

|  |  | Subtotal: |  |
| --- | --- | --- | --- |
|  | Make: |  |  |
|  | Model: |  |  |

| Pos.: | Bezeichnung | Menge | ME | Einzelpreis | Gesamtpreis |
| --- | --- | --- | --- | --- | --- |

Product-specific features

- Flush-mounted and seamless glass ceramic hob

- Thermostatic temperature control

- Electronic pan detection to save energy

- The infinitely variable setting guarantees sensitive control of the four separately controllable cooking zones.

- Large knobs with clear scales and control elements.

Housing

- Base unit : Open cupboard space closed on three sides,

- Inside base bevelled on three sides with radius at the rear.

- Completely made of CNS 1.4301

- Cover on all sides A-40

- Material thickness of side panels: 1 mm

- Cover plate material thickness: 2 mm with additional 2 mm reinforcement profiles on the underside

- visible sides in microlon finish

Equipment

- Ceramic hob, flush-mounted

- Knob control

- 4 heating zones each 300 x 300mm

- Control range: 1 - 7, infinitely variable

- 4 height-adjustable CNS feet from 894 - 930mm

- Connection cable 1.5m

| Pos.: | Bezeichnung | Menge | ME | Einzelpreis | Gesamtpreis |
| --- | --- | --- | --- | --- | --- |

Hersteller: Salvis AG, Nordstrasse 15, 4665 Oftringen, Schweiz