Tender text

Cooker Master Compact – Induction hob 4 zones full surface

| Pos.: | Description | Qty | units | price | Total |
| --- | --- | --- | --- | --- | --- |

|  | Ausführung: | Geplant: | Angeboten: |
| --- | --- | --- | --- |
|  | Type: | Free-standing unit |  |
|  | Dimensions (L x W x H):: | 800 x 800 x 750/900mm |  |
|  | Dimensions Glass (L x W): | 720 x 650mm |  |
|  | Glass thickness: | 6 mm |  |
|  | Substructure: | Storage room accessible from the front |  |
|  | Substructure Dimensions: | 320 / 378 x 759 x 468mm |  |
|  | Hygiene: | Standard |  |
|  | Substructure equipment: | Shelf, 3-way adjustable |  |
|  | Heating zone size: | 260 x 135 mm |  |
|  | Number of heating zones: | 4 |  |
|  | Electrical connected load: | 28,0 kW |  |
|  | Voltage: | 400 V 3N 50/60 Hz |  |
|  | Fuse protection: | 2 x(3 x 20 A) |  |
|  | Test and safety mark: | CE |  |
|  | Device protection type: | IPX4 |  |
|  | Sensitive heat emission: | 1,96 kW |  |
|  | Heat output latent: | 0,78 kW |  |
|  | Vapour emission: | 1,16 kg/h |  |

Spezifications

|  | 6mm glass top 720 x 650mm | No |  Yes |  |
| --- | --- | --- | --- | --- |
|  | Flush glass installation | No |  Yes |  |
|  | square coils 260 x 135 mm | No |  Yes |  |
|  | Electronic pan detection | No |  Yes |  |
|  | Thermostatic control | No |  Yes |  |
|  | Worktop 2 mm | No |  Yes |  |
|  | 2mm CNS reinforcement profiles | No |  Yes |  |
|  | 4 height-adjustable CNS feet from 894 - 930mm | No |  Yes |  |
|  | Connection cable 1.5m | No |  Yes |  |
|  | Visible sides in microlon finish | No |  Yes |  |

|  |  | Subtotal: |  |
| --- | --- | --- | --- |
|  | Make: |  |  |
|  | Model: |  |  |

| Pos.: | Description | Quantity | ME | Price | Total |
| --- | --- | --- | --- | --- | --- |

Product-specific features

- Flush-mounted and seamless glass ceramic hob

- Flush and seamless glass ceramic hob

- Electronic temperature control

- Electronic pan detection to save energy. If the cookware is removed, the heating

- automatically switched off.

- Low heat radiation and therefore reduced exhaust air heat

- The infinitely variable setting guarantees sensitive control of the four separately adjustable cooking zones.

- Large knobs with clear scales and control elements.

- Induction generator located in the substructure to protect against thermal stress

- Generator with air filter for easy cleaning

Housing

- Base unit : Open cupboard space closed on three sides,

- Inside base bevelled on three sides with radius at the rear.

- Completely made of stailess steel 1.4301

- Cover on all sides A-40

- Material thickness of side panels: 1 mm

- Cover plate material thickness: 2 mm with additional 2 mm reinforcement profiles on the underside

- visible sides in microlon finish

Equipment

- Ceramic hob, flush-mounted

- Knob control

- 4 heating zones

- Control range: 1 – 9

- Induction zones with square coils 260 x 135 mm

- 4 height-adjustable stainless steel feet from 894 - 930mm

- 2 pc Connection cable 1.5m

| Pos.: | Description | Quantity | ME | Price | Total |
| --- | --- | --- | --- | --- | --- |

Hersteller: Salvis AG, Nordstrasse 15, 4665 Oftringen, Schweiz