Tender text

Deep Fat Fryer Fryline SFR-EVO-41   Freestanding execution

| Pos.: | Description |  | QTY | Unit Price | Price Total |
| --- | --- | --- | --- | --- | --- |

|  | Execution: | Planned: | Offered: |
| --- | --- | --- | --- |
|  | Appliance type: | Freestanding execution |  |
|  | Useful filling quantity: | 10.0 – 13.5 Litre |  |
|  | Total capacity: | 17.0 – 20.5 Litre |  |
|  | Basin size: | 360 x 428 x 338 mm |  |
|  | Basket capacity: | 2.5 kg |  |
|  | Hourly output cooled/TK: | 50 kg / 38 kg |  |
|  | Dimensions (WDH): | 400 x 650 x 750/900 mm |  |
|  | Weight (net): | 70 kg |  |
|  | Capacity: | 16.0 kW |  |
|  | Voltage: | 400 V 3 |  |
|  | Frequency: | 50 / 60 Hz |  |
|  | Electrical connection: | 3LNPE |  |
|  | Fuse protection: | 3 x 25 A |  |
|  | Tests and certificates: | CE |  |
|  | Protection class: | IP X4 |  |

Specific characteristics

- Single-pan deep fat fryer for frying and baking of meat, fish, potatoes, vegetables, and pastries.

- Swivel tubular heating element in the basin with an enlarged heating surface for more efficient heat transfer, including a working and safety thermostat.

- Integrated fat/oil drain with shut-off function for safe emptying of the basin contents.

- Double filter system consisting with easy-to-clean metal and cloth filters for efficient collection of frying residues

and extended oil lifespan.

- Removable collection container made of stainless steel on ball-bearing slide rails enables safe and convenient oil and fat maintenance.

- Basin insulation (R-value 0.18 m2K/W) for minimised energy loss.

- With connection cable - WITHOUT plug.

A

F

| Pos.: | Description |  | QTY | Unit Price | Price Total |
| --- | --- | --- | --- | --- | --- |

Control:

- Simple, user-friendly interface with a high-quality membrane keypad seamlessly integrated into the control panel.

- Digital display for optimal readability of temperature and time.

- Four buttons for easy selection of variable temperature and time settings.

- Automatic fat melting stage during the heating phase.

- Programmable fat/oil usage duration for optimal monitoring of fat lifespan.

- AQA – Automatic Quantity Adjustment: Intelligent quantity adaptation for optimal cooking results.

- Energy-saving function - automatic temperature reduction after 30 minutes of inactivity.

- Electronic timer with with start/stop button and acoustic signal.

- Fat temperature adjustable from 50 - 180 °C for increased performance and capacity.

- Precise temperature detection of +/-1 °C thanks to electronic temperature sensor.

Housing:

- Seamless and deep drawn and joint less welded basin with large scaled radiuses, for easy cleaning.

- Wide foaming zone and drain downwards throughout a ball-valve.

- Closed base unit with hinged door, left-side hinge – easily changeable to the right.

- Adjustable CNS feet, 150 mm high

- Complete unit, basin, and base unit cladding made of CNS 1.4301 stainless steel.

Options and accessories

- Mobile kit, 2 adjustable legs in front and 2 fixed castors on the rear, 1 pan, with 1 cable,

with 1 plug type CEE 32 A

- Frying basket 200, 140 x 320 x 155 mm (2 baskets can be placed in one basin)

- Frying basket portion, 105 x 320 x 155 mm (2 baskets can be placed in one basin)

- Lid 400 for version with 2 portion baskets

- Connecting path for hygienic and safe installation of 2 free-standing fryers

- MAMITO - Automatic dosing system to extend the oil service life

Technical subjects to modification!

Manufacturer: Salvis AG, Nordstrasse 15, 4665 Oftringen, Switzerland